Sound and Music File Formats

A. Formats based on sound waveforms

These formats are essentially digital sound recordings - the digital equivalents of tape or disk recordings. You can easily make your own digital sound recordings using only a microphone and a sound recorder or editor program.  A digital sound recording may be mono or stereo, have 8 or 16 bits, and may have any of several sampling rates.  When you digitize your own sounds using a sound recorder program,  you can choose these parameters.  Each of these have an influence on the size of the resulting computer file and on the quality of the sound.  Stereo recordings have two channels (left and right) and take up about twice the disk space of a mono (single channel) recording.  A 16 bit recording takes about twice the disk space of an 8-bit recording, but the sound quality and dynamic range is better.  The most common sampling rates are 11, 22, and 44 KHz; the larger the sampling rate, the larger the file size and the better the audio fidelity.  For the best sound quality for music, use 16 bits and 44 KHz. Voice recordings can get by with 22 or even 11 KHz in order to produce smaller files.

Non-compressed formats: some examples are WAV, AU, and AIFF.  These files are often very large.  A 3-minute song of CD-quality music (stereo, 16 bits, 44 KHz sampling rate) may take up 30 Megabytes.  These file types can be played by virtually all current sound players.  A sound editor program such as Cool Edit can read and write and convert between these formats.

MP3 (compressed) format:  This format uses compression to reduce the file size of a digitized recording without degrading the audio quality.  A 3-minute song song of CD-quality music (stereo, 16 bits, 44 KHz sampling rate) may be only about 3 Megabyte MP3 file.  Some recorder programs that can record sound files directly in MP3 format or can convert non-compressed sound files into MP3.  You can also use a CD-recorder program (such as MusicMatch) to automatically read an audio CD and convert its tracks into MP3 files and save them on your hard disk. MP3 files can be played by your computer's sound player or on portable MP3 players.

B. Formats based on musical notes

These formats specify the music notes played in a musical performance.   They are the digital equivalents of sheet music; they do not record the actual sound of a particular performance. These formats are used mainly for instrumental music.

MIDI format (file extension ".MID"): This is the most generic note-based format. It includes the notes, their duration and loudness, and the instuments used. MIDI files are very compact compared to the sound-based files of the same music - much smaller than any other format; a 3-minute song may be only a few Kbytes (v.s. a few Mbytes as an MP3 file). MIDI files can be played by most sound players of contemporary computers (e.g. MediaPlayer, Quicktime, WinAmp).  The sound quality depends on the quality of the computer's sound card. Very old computers may not be able to play MIDI files. Some MIDI players allow you to change the tempo, musical key, and the instruments assigned to each part.  To record a MIDI file, you need to have a MIDI-capable musical instrument, e.g. a keyboard, drum pad, or other instrument that can hook up to a computer running a MIDI sequencer program. With good-quality MIDI instruments, every nuance of a performace is recorded into the MIDI file: the exact phrasing, dynamics, and expression of each individual note, the action of the sustain pedals, pitch bends, etc.

Score editor formats: These are the proprietary formats of score-editor programs such as Finale. In addition to the notes and durations, these formats include the details of score typography (staves, clefs, stem directions, joined note tails, etc.).  Using a score editor program, a musician can write music from scratch, then hear it played by the computer, print out the score parts, and save it as a MIDI file. However, a MIDI file created from a score will not have all the performance nuances of a live MIDI recording. For example, in the "Sound file examples" folder, compare "Moonlight1.mid", created from the score, to "Moonlight2.mid", created from a live recording on a MIDI piano. The notes and the tempo are exactly the same; can you hear a difference? Listen carefully - one sounds more mechanical and lifeless; the other sound more alive.

KAR format: A specialized format used for karaoke; it includes the song lyrics and the music accompaniment, often in MIDI format.  A MIDI-karaoke player application (e.g. WinKaraoke in the "Windows software" folder) will read this format and display the lycics synchronized with the music accompaniment.

(c) Tom O'Haver, revised February 2004